# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Supply of laboratory and metrology equipment for testing and calibration laboratories in Ghana

# Publication reference: EuropeAid/133510/D/SUP/GH

Column 1-2 should be completed by the Contracting Authority

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

LOT 12: ASSORTED ELECTRONIC EQUIPMENT

| 1. Item Number | 2. Specifications Required (minimum) | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 078 | Computer, laptop - Processor: equivalent to Intel-i7, 3rd generation, - Internal memory: at least 4 Gb - Hard drive: at least 500 Gb HHD and/or SSD, and DVD ± R reader/writer - Keyboard with enhanced keyboard layout (minimum 101 keys), QWERTY/US International, and wireless mouse - Display: Minimal approx. 15” and maximum approx. 17”, high definition LED, resolution at least 1280 x 1024, with guarantee for zero defect pixels and sub-pixels for 30 days, thereafter according to ISO 13406-2 Class II for 3 years - Communication: WIFI (802.11 b/g/n), BT, at least 3 USB slots, RJ-45, and HDMI - Webcam, speakers, and microphone integrated - Lithium-ion battery with minimal 3 hours operating time, AC adapter/charger with cord, 220-240 V, 50 MHz, British plug BS-1363 or suitable adapter - Windows 8 (64 bit) or equivalent - Software for word-processing, spreadsheets, email, presentation, and internet browser, compatible with the latest Microsoft Office programmes (2013 version) - Virus protection and internet security software with subscription for 3 years - Carrying bag with ample room for cords and other accessories |  |  |  |
| 079 | Computer, desk top - Processor: equivalent to Intel-i7, 3rd generation, - Internal memory: at least 8 Gb - Hard drive: at least 500 Gb HHD and/or SSD, and DVD ± R reader/writer - Keyboard with enhanced keyboard layout (minimum 101 keys), QWERTY/US International, and wireless mouse - Colour monitor at least 20" flat widescreen with guarantee for zero defect pixels and sub-pixels for 30 days, thereafter according to ISO 13406-2 Class II for 3 years - Communication: WIFI (802.11 b/g/n), BT, at least 3 USB slots, RJ-45, and HDMI - Webcam, speakers, and microphone integrated - Windows 8 (64 bit) or equivalent - Software for word-processing, spreadsheets, e-mail, presentation, and internet browser, compatible with the latest Microsoft Office programmes (2013 version) - Virus protection and internet security software with subscription for 3 years - Supplied with a UPS for 15 minutes |  |  |  |
| 080 | Printer - Laser technology - Black and white printing only - 600 x 600 dpi print resolution - Output: minimum 10 ppm (4) - Automatic duplex printing - Minimum 150 sheet capacity input tray, and 50 sheet output tray - Memory: 32MB RAM (expandable), or higher - Connectivity: Ethernet 10/100Base-TX, USB 2.0 high speed [wireless connection optional: 802.11b/g/n] - Compatible with Windows 8 - Low energy consumption - 2 additional high capacity toner cartridges |  |  |  |
| 081 | Multifunctional printer - Laser technology - Functions: printer, copier, scanner (fax optional) - 600 x 600 dpi print resolution - Black and white printing (colour optional) - Output: minimum 10 ppm (A4) b/w, (and 4 ppm colour, if colour option is offered) - Minimum 150 sheet capacity input tray, and 50 sheet output tray - Automatic duplex printing and copying - Automatic document feeder (ADF) with minimum 25-sheet capacity - Memory: minimal 64 MB RAM (expandable) - Connectivity: Ethernet 10/100Base-TX, USB 2.0 high speed (IEEE 802.11b/g/n wireless connection optional) - Scanner: colour - Scan formats: minimal TIFF, JPEG, PDF - Compatible with Windows 8 - Additional high capacity toner cartridges (2 sets) |  |  |  |
| 084 | GPS, handheld - To be used for mapping sites of interest for plant protection - Trekking type - Map of Ghana |  |  |  |
| 130 | Digital camera - Compact digital camera - Resolution: 15 mega-pixel, or better - Autofocus - 4x optical zoom lens or better - Macro mode - Built in GPS for determination of position - Minimum 4 GB memory card - A mini tripod with a multi functional 3D panoramic head and extendable legs up to approx. 40 - 60 cm - With a carrying case or bag for protection of the camera with storage room for charger, tripod and other accessories |  |  |  |
| 199 | Calculator - Handheld calculator - basic functions: addition, subtraction, division and multiplication. - memory functions - LCD display: minimum 10+2 digits - Battery operated (optional in combination with solar panel) |  |  |  |
| 407 | UPS 1 kVA UPS 1 kVA, 15 minutes at rated power - Features: - Provide a battery backed up supply to keep equipment operating when there is a power cut - Fluctuations in voltage automatically corrected, without using the batteries - Surge protection: high performance surge protection compliant with IEC 616431 standard - Power management system: Indication of Low Battery, On Bypass, Summary Alarm, Input Fail, UPS On - Wide tolerance of input voltage to avoid frequent changeover to battery power - Adjustable thresholds for changeover to battery power - Specifications: - Input: - Voltage Range: single phase, 230 V, 50 Hz, -20% to +15% - Current: Sinusoidal, power factor 0.95 PF under all line/load conditions - Efficiency: > 90 % - Output: - Voltage: single phase, 230 V, 50 Hz - Load capacity: 1 kVA, 0.7 kW - Voltage Regulation: ± 3% No Load to Full Load, High Line to Low Line - Frequency: 50 Hz ± 0.5 Hz (when on inverter) - Wave Form: Sine Wave - Harmonic Distortion: <5 % THD; <3 % Single Harmonic - Crest Factor: 3 to 1 - Changeover from mains to battery or back: < 20 ms (1 cycle) - Overload: 125 % for 10 minutes, 150 % Surge for 10 seconds - Protection: Internal electronic overload protection and circuit breaker to provide inherent overload protection - Batteries: - Run Time: 15 minutes at rated maximum load - Battery Type: Sealed, Maintenance-Free, Long Life - Battery life: 10 years typical - Charger current: < 10 times discharge current - Charging time: < 8 hours - Including: - Connection to PC (via network or dedicated cable), isolated from mains, for monitoring and control - Power management software - Installation and testing - User manuals in English - Service/repair contract and spares for 3 years |  |  |  |
| 408 | UPS 2 kVA UPS 1 kVA, 15 minutes at rated power - Features: - Provide a battery backed up supply to keep equipment operating when there is a power cut - Fluctuations in voltage automatically corrected, without using the batteries - Surge protection: high performance surge protection compliant with IEC 616431 standard - Power management system: Indication of Low Battery, On Bypass, Summary Alarm, Input Fail, UPS On - Wide tolerance of input voltage to avoid frequent changeover to battery power - Adjustable thresholds for changeover to battery power - Specifications: - Input: - Voltage Range: single phase, 230 V, 50 Hz, -20% to +15% - Current: Sinusoidal, power factor 0.95 PF under all line/load conditions - Efficiency: > 90 % - Output: - Voltage: single phase, 230 V, 50 Hz - Load capacity: 2 kVA, 1.4 kW - Voltage Regulation: ± 3% No Load to Full Load, High Line to Low Line - Frequency: 50 Hz ± 0.5 Hz (when on inverter) - Wave Form: Sine Wave - Harmonic Distortion: <5 % THD; <3 % Single Harmonic - Crest Factor: 3 to 1 - Changeover from mains to battery or back: < 20 ms (1 cycle) - Overload: 125 % for 10 minutes, 150 % Surge for 10 seconds - Protection: Internal electronic overload protection and circuit breaker to provide inherent overload protection - Batteries: - Run Time: 15 minutes at rated maximum load - Battery Type: Sealed, Maintenance-Free, Long Life - Battery life: 10 years typical - Charger current: < 10 times discharge current - Charging time: < 8 hours - Including: - Connection to PC (via network or dedicated cable), isolated from mains, for monitoring and control - Power management software - Installation and testing - User manuals in English - Service/repair contract and spares for 3 years |  |  |  |
| 409 | UPS 5 kVA UPS 1 kVA, 15 minutes at rated power - Features: - Provide a battery backed up supply to keep equipment operating when there is a power cut - Fluctuations in voltage automatically corrected, without using the batteries - Surge protection: high performance surge protection compliant with IEC 616431 standard - Power management system: Indication of Low Battery, On Bypass, Summary Alarm, Input Fail, UPS On - Wide tolerance of input voltage to avoid frequent changeover to battery power - Adjustable thresholds for changeover to battery power - Specifications: - Input: - Voltage Range: single phase, 230 V, 50 Hz, -20% to +15% - Current: Sinusoidal, power factor 0.95 PF under all line/load conditions - Efficiency: > 90 % - Output: - Voltage: single phase, 230 V, 50 Hz - Load capacity: 5 kVA, 3.5 kW - Voltage Regulation: ± 3% No Load to Full Load, High Line to Low Line - Frequency: 50 Hz ± 0.5 Hz (when on inverter) - Wave Form: Sine Wave - Harmonic Distortion: <5 % THD; <3 % Single Harmonic - Crest Factor: 3 to 1 - Changeover from mains to battery or back: < 20 ms (1 cycle) - Overload: 125 % for 10 minutes, 150 % Surge for 10 seconds - Protection: Internal electronic overload protection and circuit breaker to provide inherent overload protection - Batteries: - Run Time: 15 minutes at rated maximum load - Battery Type: Sealed, Maintenance-Free, Long Life - Battery life: 10 years typical - Charger current: < 10 times discharge current - Charging time: < 8 hours - Including: - Connection to PC (via network or dedicated cable), isolated from mains, for monitoring and control - Power management software - Installation and testing - User manuals in English - Service/repair contract and spares for 3 years |  |  |  |

Requested training:

| Spec | Equipment | Training |
| --- | --- | --- |
|  | No training required for Lot 12 |  |

General Remarks:

- If calibration is required, the calibration certificate must be issued by a calibration laboratory accredited by an accreditation body, which is a signatory of the Mutual Recognition Arrangement of the International Laboratory Accreditation Cooperation (ILAC-MRA).

- All electrical equipment should be suitable for operating on 220-240V / 50-60 Hz (unless otherwise specified), and provided with a BS-1363 plug, or with a suitable adapter.

- Software that is required for the realisation of functionalities of equipment must be the latest version, and should be compatible with a Windows 8 or equivalent operating system. Software updates should be provided for free during at least for a period of 36 months.

- For computers/system controllers and related supplementary equipment (e.g. flat screen, printer, UPS) that are considered to be an integrated part of the equipment configuration and that are not further specified, the contractor will provide systems of good quality and reputable brands, based on latest technology, and latest software for operating systems (Windows 8, or equivalent), for office applications (MS Office, or equivalent), and anti-virus protection at least for a period of 36 months